Introduction to IoT

The Internet of Things (IoT) refers to a network of physical objects—"things"—embedded with sensors, software, and other technologies to connect and exchange data with other devices and systems over the internet.

Examples of IoT Devices:

- Smart thermostats

- Wearable fitness trackers

- Connected home appliances

- Industrial sensors

Components of IoT

1. Sensors / Devices

These collect data from the environment.
Examples: temperature sensors, motion sensors, light sensors.

2. Connectivity

Enables data transfer to the cloud or other devices.
Technologies: Wi-Fi, Bluetooth, GSM, Zigbee, LoRa, etc.

3. Data Processing

Processes the collected data to make decisions.
This may happen on:

- The microcontroller itself (local or edge computing)

- A server or cloud platform (remote processing)
Example: If motion is detected → Turn on a light.

4. User Interface

Allows users to interact with the system.
Examples:

- A mobile app showing temperature

- A web dashboard to monitor data or control devices

Essential Hardware Components

- Microcontrollers:

  - E.g., Arduino Uno, ESP8266; act as the "brains" of IoT devices.

- Sensors:

  - Detect physical and environmental changes.

  - Examples:

    - Motion sensors (e.g., PIR sensors)

    - Temperature sensors (e.g., DHT11)

    - Light sensors (e.g., LDRs)

- Actuators:

  - Perform actions in response to processed data.

  - Examples:

    - Buzzers sounding an alarm

    - LEDs indicating system status

- Communication Modules:

  - Enable data transmission.

  - Examples:

    - Wi-Fi modules (e.g., ESP8266)

    - GSM modules for SMS alerts

Common Boards and Their Pinouts

      - ESP8266 (NodeMCU)

Overview: The ESP8266 is a low-cost Wi-Fi microchip with full TCP/IP stack and microcontroller capability. The NodeMCU is a development board that incorporates the ESP8266.

Pinout Highlights:

- Digital I/O Pins (D0–D8): Used for digital input or output.

- Analog Input (A0): Used to read analog signals (0–1V).

- Power Pins:

  - Vin: Can be used to supply voltage to the board.

  - 3V3: Provides 3.3V output.

  - GND: Ground pins.

      - Arduino Uno

- Digital I/O Pins (0–13): Used for digital input or output. Pins 0 (RX) and 1 (TX) are also used for serial communication.

- Analog Input Pins (A0–A5): Used to read analog signals.

- PWM Pins: Pins 3, 5, 6, 9, 10, and 11 support Pulse Width Modulation.

- Power Pins:

  - 5V: Provides regulated 5V output.

  - 3.3V: Provides 3.3V output.

  - GND: Ground pins.

- Reset Pin: Used to reset the microcontroller.

        - Raspberry Pi

The Raspberry Pi (models B+, 2, 3, 4, and 5) features a 40-pin header that allows for interfacing with various electronic components. These pins are categorized based on their functions:

- Power Pins: Provide voltage to external components.

- Ground Pins: Serve as the common ground reference.

- GPIO Pins: Configurable as input or output for general-purpose use.

- Special Function Pins: Support specific protocols like I2C, SPI, UART, and PWM.

Example Use Case: ESP8266 (NodeMCU) to control an LED via a web browser.

Components Needed:

- NodeMCU ESP8266 (e.g., ESP-12E)

- LED (any color)

- 220Ω Resistor

- Breadboard and Jumper Wires

- Micro USB Cable (to connect NodeMCU to your computer)

Circuit Diagram:

Connections:

- LED Anode (+) → NodeMCU D1 pin (GPIO5)

- LED Cathode (-) → Ground (GND) through a 220Ω resistor

Programming the NodeMCU:

- Install the Necessary Libraries:

  - Open the Arduino IDE.

  - Go to Sketch → Include Library → Manage Libraries.

  - Search for and install the ESP8266 board package.

- Select the NodeMCU Board:

  - Go to Tools → Board → NodeMCU 1.0 (ESP-12E Module).

- Write the Code:

#include <ESP8266WiFi.h>

const char* ssid = "Your_SSID";

const char* password = "Your_PASSWORD";

WiFiServer server(80);

void setup() {

pinMode(D1, OUTPUT);

Serial.begin(115200);

delay(10);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(1000);

Serial.println("Connecting to WiFi...");

}

server.begin();

Serial.println("Server started");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

}

void loop() {

WiFiClient client = server.available();

if (!client) {

return;

}

String request = client.readStringUntil('\r');

client.flush();

if (request.indexOf("/LED=ON") != -1) {

digitalWrite(D1, HIGH);

} else if (request.indexOf("/LED=OFF") != -1) {

digitalWrite(D1, LOW);

}

client.print("HTTP/1.1 200 OK\r\n");

client.print("Content-Type: text/html\r\n");

client.print("\r\n");

client.print("<!DOCTYPE HTML>\r\n");

client.print("<html>\r\n");

client.print("<body>\r\n");

client.print("<h1>LED Control</h1>\r\n");

client.print("<p><a href=\"/LED=ON\">Turn On</a></p>\r\n");

client.print("<p><a href=\"/LED=OFF\">Turn Off</a></p>\r\n");

client.print("</body>\r\n");

client.print("</html>\r\n");

delay(1);

client.stop();

}

- Upload the Code:

  - Connect the NodeMCU to your computer via the micro USB cable.

  - Select the correct port from Tools → Port.

  - Click the Upload button in the Arduino IDE.

Accessing the Web Server:

- Find the NodeMCU's IP Address:

  - After uploading the code, open the Serial Monitor in the Arduino IDE.

  - Set the baud rate to 115200.

  - Note the IP address displayed in the Serial Monitor (e.g., 192.168.1.100).

- Control the LED:

  - Open a web browser and enter the NodeMCU's IP address:

http://192.168.1.100

  - You should see a simple webpage with two links: "Turn On" and "Turn Off".

  - Clicking "Turn On" will light up the LED, and "Turn Off" will turn it off.

IoT Security Challenges

- Insecure interfaces (e.g., default credentials)

- Unencrypted data transmission

- Lack of firmware updates

- Physical tampering with devices

Mitigations:

- TLS/SSL, firmware signing, authentication, secure boot